STM32F407 DISCOVERY BOARD PROGRAMMING ASSIGNMENT

Assignment 5: SPI and LIS3DSH Accelerometer

Assumption: Use the STM32F407G-DISC1 board and STM32 HAL. Use SPI1 to communicate with the onboard LIS3DSH accelerometer. Keep SWD enabled for debugging.

Board Connections:

SPI1_SCK  : PA5

SPI1_MISO : PA6

SPI1_MOSI : PA7

LIS3DSH CS: PE3

LIS3DSH INT1/DRDY: PE0

LIS3DSH INT2: PE1

Onboard LEDs:

Green  LD4: PD12

Orange LD3: PD13

Red    LD5: PD14

Blue   LD6: PD15

For this assignment, configure SPI1 as an 8-bit, full-duplex master using software NSS, MSB-first data, SPI mode 3, and a safe clock frequency such as 1 MHz. SPI mode 3 uses CPOL = 1 and CPHA = 2EDGE.

Important LIS3DSH Registers:

configuration registers:-

WHO_AM_I  : 0x0F, expected value 0x3F

CTRL_REG4 : 0x20, output data rate, block-data update, and axis enable

CTRL_REG3 : 0x23, 1: data ready signal connected to INT1, 1: interrupt signals active HIGH, 1: INT1/DRDY signal enabled

CTRL_REG5 : 0x24, filter bandwidth, full-scale range, self-test, and SPI mode

control registers:-

OUT_X_L   : 0x28

OUT_X_H   : 0x29

OUT_Y_L   : 0x2A

OUT_Y_H   : 0x2B

OUT_Z_L   : 0x2C

OUT_Z_H   : 0x2D

Q1. SPI Configuration and Transmit/Receive Loopback Test

Configure SPI1 and verify the STM32 SPI transmit and receive paths using a physical loopback connection.

For the loopback test, temporarily connect PA7 (SPI1_MOSI) to PA6 (SPI1_MISO) using a jumper wire. Keep the LIS3DSH chip-select pin PE3 high so that the sensor is not selected.

Tasks:

1. Enable the clocks for SPI1, GPIOA, and GPIOE.

2. Configure PA5 as SPI1_SCK using the correct alternate-function setting.

3. Configure PA6 as SPI1_MISO using the correct alternate-function setting.

4. Configure PA7 as SPI1_MOSI using the correct alternate-function setting.

5. Configure PE3 as a push-pull output for LIS3DSH chip select.

6. Set PE3 high when the sensor is not being accessed.

7. Configure SPI1 as an 8-bit full-duplex master.

8. Select software NSS, MSB-first data, SPI mode 3, and a suitable prescaler.

9. Transmit and receive test patterns such as 0x55, 0xAA, 0x00, and 0xFF.

10. Use HAL_SPI_TransmitReceive() for the loopback test.

11. Compare each received byte with the transmitted byte.

12. Indicate loopback success and failure using different onboard LEDs.

13. Remove the loopback wire before testing the LIS3DSH.

14. Keep SWD enabled for debugging.

Q2. Configure the LIS3DSH for Continuous Measurement

Configure the LIS3DSH to measure the X, Y, and Z axes continuously.

Read the six accelerometer output registers continuously and combine the received bytes into signed 16-bit X, Y, and Z values.

Detect Board Tilt and Control the Onboard LEDs

Use the accelerometer readings to determine the board tilt direction and turn on a different onboard LED for each direction.

Tasks_1:

1. Implement an SPI register-read function for the LIS3DSH.

2. Check the STATUS register before reading data, or use a suitable sampling delay for the configured output data rate.

3. Start a multiple-byte read at OUT_X_L (0x28).

4. Enable register address auto-increment for the six-byte read.

5. Read OUT_X_L, OUT_X_H, OUT_Y_L, OUT_Y_H, OUT_Z_L, and OUT_Z_H.

6. Combine each low byte and high byte into a signed 16-bit value.

7. Store the results in signed variables named accel_x, accel_y, and accel_z.

8. Display the raw values using a debugger or serial terminal.

Tasks_2:

1. Configure PD12, PD13, PD14, and PD15 as push-pull outputs.

2. Define a tilt threshold to prevent LED changes caused by small sensor noise.

3. Compare the absolute X, Y and Z values to determine the dominant tilt axis.

4. Turn on only the LED associated with the detected direction.

5. Turn off all LEDs when the board is approximately level or below the threshold.

6. Add a small delay or hysteresis so that the LEDs do not flicker near the threshold.

7. Record the raw accelerometer values observed for each direction.

8. Keep the tilt detection and LED control running continuously.

9. Keep SWD enabled for debugging

References:

LIS3DSH Datasheet:

https://www.st.com/resource/en/datasheet/lis3dsh.pdf