STM32F407 DISCOVERY BOARD PROGRAMMING ASSIGNMENT

Assignment 6: I2C Communication Between Two STM32F4 Boards

Assumption: Use two STM32F4 boards with STM32 HAL. Board 1 is the I2C master and Board 2 is the I2C slave. Use polling mode.

Hardware Connections:

Board 1 PB6 (SCL)  ----------------  Board 2 PB6 (SCL)

Board 1 PB9 (SDA)  ----------------  Board 2 PB9 (SDA)

Board 1 GND        ----------------  Board 2 GND

Use open-drain GPIO configuration and external pull-up resistors on SCL and SDA.

I2C Configuration:

Peripheral: I2C1

SCL: PB6

SDA: PB9

Speed: 400 kHz Fast Mode

Duty cycle: I2C_DUTYCYCLE_16_9

Addressing mode: 10-bit

Slave address: 0x001D

Onboard LEDs:

LED1 Green:  PD12

LED2 Orange: PD13

LED3 Red:   PD14

LED4 Blue:  PD15

User button: PA0

Q1. Configure I2C1 on Both Boards

Configure the GPIO pins and I2C1 peripheral on both STM32F4 boards.

Tasks:

1. Create one master project and one slave project.

2. Enable the clocks for GPIOA, GPIOB, GPIOD, and I2C1.

3. Configure PB6 as I2C1_SCL using Alternate Function 4.

4. Configure PB9 as I2C1_SDA using Alternate Function 4.

5. Configure PB6 and PB9 as open-drain pins.

6. Configure PD12, PD13, PD14, and PD15 as LED outputs.

7. Configure PA0 as a user-button input on the master board.

8. Configure I2C1 for 400 kHz Fast Mode.

9. Select I2C_DUTYCYCLE_16_9.

10. Select 10-bit addressing and slave address 0x001D.

11. Initialize I2C1 using HAL_I2C_Init().

12. Keep SWD enabled for debugging.

Q2. Send LED Status from the Master Board

When the user presses the master-board button, transmit a two-byte data packet to the slave board.

Packet format:

aTxBuffer[0] = LED status number

aTxBuffer[1] = 0xAA

LED status values:

0: All LEDs OFF

1: Green LED ON

2: Orange LED ON

3: Red LED ON

4: Blue LED ON

Tasks:

1. Declare a two-byte transmit buffer named aTxBuffer.

2. Read the user button using HAL_GPIO_ReadPin().

3. Add a 50 ms delay after detecting a button press.

4. Increase the LED status number from 1 to 4 and then return to 0.

5. Store the LED status in aTxBuffer[0].

6. Store 0xAA in aTxBuffer[1].

7. Transmit both bytes using HAL_I2C_Master_Transmit().

8. Wait for the button to be released.

9. Add a short delay before the next transmission.

10. Keep the master operation in polling mode.

Q3. Receive the LED Status on the Slave Board

Receive the two-byte packet on the slave board and use the first byte to control the LEDs.

Tasks:

1. Declare a two-byte receive buffer named aRxBuffer.

2. Receive two bytes using HAL_I2C_Slave_Receive().

3. Pass aRxBuffer[0] to an LED_Display() function.

4. Turn off all LEDs at the beginning of LED_Display().

5. Turn on PD12 when the status is 1.

6. Turn on PD13 when the status is 2.

7. Turn on PD14 when the status is 3.

8. Turn on PD15 when the status is 4.

9. Leave all LEDs off when the status is 0.

10. Continue waiting for the next packet.

11. Keep the slave operation in polling mode.

Q4. Display the Received LED Sequence

Demonstrate the transmission and reception of the LED status sequence.

Tasks:

1. Program Board 1 as the master.

2. Program Board 2 as the slave.

3. Connect PB6, PB9, and GND between the boards.

4. Press the master-board button once and turn on the green LED on the slave.

5. Press the button again and turn on the orange LED on the slave.

6. Press the button again and turn on the red LED on the slave.

7. Press the button again and turn on the blue LED on the slave.

8. Press the button again and turn off all LEDs on the slave.

9. Observe aTxBuffer and aRxBuffer in the debugger.

10. Record the LED status sequence displayed by the slave.

Q5. Explain Polling-Mode I2C Buffer Transfer

Explain how the two boards exchange the LED status packet.

Tasks:

1. Explain the role of the I2C master.

2. Explain the role of the I2C slave.

3. Explain the purpose of aTxBuffer[0].

4. Explain the purpose of aTxBuffer[1].

5. Explain how the slave receives the bytes into aRxBuffer.

6. Explain why PB6 and PB9 are connected between both boards.

7. Explain why the boards require a common ground.

8. Explain the difference between polling mode and interrupt mode.

9. Keep SWD enabled for debugging.

HAL APIs to Use:

HAL_I2C_Init()

HAL_I2C_Master_Transmit()

HAL_I2C_Slave_Receive()

HAL_GPIO_ReadPin()

HAL_GPIO_WritePin()

HAL_Delay()